Natural Resources Conservation Service (NRCS)

CP-21Filter Strips (ver. Minn. 5/10)

Landowner:     .

Definition

A filter strip is an area of permanent herbaceous vegetation used to reduce sediment, organics, nutrients, pesticides, and other contaminant loadings in runoff.

Purpose

Filter strips provide a buffer between fields and water bodies and allow for settling out of suspended soil particles, infiltration of runoff and soluble pollutants, adsorption of pollutants on soil and plant surfaces, and uptake of soluble pollutants by plants.  Filter strips can also restore, create or enhance herbaceous habitat for wildlife and beneficial insects.

Where used

- Along streams, lakes, ponds, open tile inlets, sinkholes and select wetlands and drainageways.

- Where uniform shallow runoff can enter the strip (avoid concentrated flow).

- Where they can be installed on the approximate contour.

Conservation management system

Filter strips are effective when used in combination with other conservation practices that reduce soil, nutrient, pesticide and other material movement on the area contributing runoff to the strip.

Specifications

Your site-specific filter strip designs are shown on the attached specifications sheet and if needed job sketch sheet.  The designs were developed in accordance with Minn. NRCS Conservation Practice Standard (393) Filter Strip.

Operation, maintenance and management

- Control all noxious weeds as identified by state and local laws, by:  (1) treating with chemicals per label directions, or (2) spot mow before seed heads form.  When possible delay use of control measures until after August 1st to protect nesting wildlife.  Spot mowing must be authorized by FSA during the primary nesting season.

- Control rodent infestations that adversely affect the ground cover or the ability to carry out management activities.

- Prevent dead furrows from forming along the edge of the filter strip.

- Protect the area from haying or grazing.  Fences may need to be constructed and maintained to exclude livestock throughout all 12 months of each year.

- Avoid direct spray application and spray drift when applying pesticides on adjacent cropland.

- Do not use the filter strip for field roads, turn areas or other uses that will damage or destroy cover.

- Do not apply animal or other organic waste.

- Annually inspect the filter strip and repair any gullies that have formed.

- Periodically remove unevenly deposited sediment accumulation that disrupts sheet flow and re-level the filter strip and filter strip-field interface.  Reseed the leveled area if necessary or other areas that do not have adequate permanent cover.

Mid-contract Management

- Implement one of the following mid contact management activities:

- Mowing

  - Adjust cutting to leave a stubble height of 6 inches for cool season grasses and 10 inches for warm season grasses.  Cut between August 1 and September 1.

  - Use a flail chopper or rotary mower to evenly distribute the vegetation. Do not leave windrows or clipping piles that will smother the vegetation.  Clippings may also be baled, removed from the field and destroyed according to FSA requirements.

- Burn and inter-seed

  - Refer to NRCS in Minnesota Mid-contract documents  “Conservation Reserve Program-Grassland Management” for prescribed burning guidance and “Conservation Reserve Program-  Forb and Legume Inter-seeding For Wildlife” for inter-seeding guidance

  - By entering into this cost-share agreement, landowners acknowledge that the Natural Resources Conservation Service (NRCS) has informed me that I am responsible for all liability associated with this practice, and hereby release the Government and its agents and employees from any responsibility or legal liability arising from injury or harm to person or property that occurs as a result of this practice.  Landowners acknowledge that they knowingly assume all risk of injury while participating in this agreement.

- Optional Brush Management

    - Mechanical or manual control methods are preferred. The only chemical control methods allowed are “cut stump” and “basal bark”.

    - Refer to NRCS in Minnesota “Conservation Reserve Program-Mid-Contract Brush Management” for guidance.

Site Information

Sensitive Area to be Protected: OR Planned RUSLE2 Soil Loss on Contributing Area: Predominate % slope on Contributing Area: OR Size of Area Contributing uniform sheet flow to Filter Strip: Planned Ratio of Contributing Area Size to Filter Strip Size: OR Planned Life Expectancy of Filter Strip if ratio exceeds 60:1 or predominate slope exceeds 12%:

| ## Filter Strips – Specifications Sheet (MN 5/10) CP-21 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Landowner |  |  |  | Tracts |  |  |  | Fields |  |  |  |
| Filter Strip Layout | Filter Strip 1 |  |  |  | Filter Strip 2 |  |  |  | Filter Strip 3 |  |  |
| Width (ft.) Max. Min. Avg. |  |  |  |  |  |  |  |  |  |  |  |
| Length (ft.) |  |  |  |  |  |  |  |  |  |  |  |
| Area (ac.) |  |  |  |  |  |  |  |  |  |  |  |
| Slope (%) of filter strip |  |  |  |  |  |  |  |  |  |  |  |
| Species #1, variety and seeding rate in lb./acre pure live seed (PLS)* |  |  |  |  |  |  |  |  |  |  |  |
| Species #2 , variety and seeding rate in lb./acre PLS |  |  |  |  |  |  |  |  |  |  |  |
| Species #3, variety and seeding rate in lb./acre PLS |  |  |  |  |  |  |  |  |  |  |  |
| Companion crop and seeding rate (lb./acre PLS) |  |  |  |  |  |  |  |  |  |  |  |
| Seeding dates |  |  |  |  |  |  |  |  |  |  |  |
| Lime (tons/acre) |  |  |  |  |  |  |  |  |  |  |  |
| N (lb./acre) |  |  |  |  |  |  |  |  |  |  |  |
| P2O5 (lb./acre) |  |  |  |  |  |  |  |  |  |  |  |
| K2O (lb./acre) |  |  |  |  |  |  |  |  |  |  |  |

*Note:  pure live seed (PLS) is lbs. of bulk seed x %germination x %purity.  To obtain pounds of bulk seed needed per acre, use the following:  (lbs./acre PLS) divided by (%germination x  %purity).

Width refers to flow length measured perpendicular to the prevailing contour.

Length is measured with the prevailing contour or parallel to either the feature being protected (e.g. stream) or field edge.

| Practices used to convert channelized flow to shallow sheet flow |  |  |
| --- | --- | --- |
| The following practices will be installed to convert channelized runoff into overland sheet flow: |  |  |
| Site Preparation |  |  |
| Grade, smooth or fill filter strip area prior to seeding to eliminate rills and ephemeral gullies. Prepare a firm seedbed. For conventional seeding the seedbed should be worked to a depth of 3 inches and also rolled or cultipacked prior to broadcast seeding operations. Lime and fertilize according to above recommendations. |  |  |
| Seeding Methods |  |  |
| Drill grass and legumes to a depth of 1/8 to ¼ inches for conventional seeding and ¼ to ½ inches for no-till plantings. Broadcast seedings should be rolled or cultipacked immediately after seeding. If necessary, mulch seeded area with tons per acre of mulch material. |  |  |
| ### Additional Establishment Specifications |  |
| Clip or harvest companion crop after jointing but before heading. Chop rather than swath if clippings will not be removed. |  |
| Clip or chemically control annual weeds and other competition before seed heads appear in the 1st two years of establishment. Delay clipping as long as possible to protect wildlife and do not clip cool season grasses after Sept. 1 and warm season grasses after August 1. Adjust clipping height to leave a stubble height of 6 inches for cool season grasses and 10 inches for warm season grasses. Clip in a manner that prevents a mat that will smother the vegetation. |  |

Additional Operation and Maintenance Instructions or Comments

| Landowner: |  | FSA Contract Number |
| --- | --- | --- |
| Prepared by: |  | Planned Application Date |
| Tract Number(s) | Field Number(s) | Total Acres |

| Total Acres | X cost/acre | = Project Cost Estimate | x Cost Share Rate |
| --- | --- | --- | --- |
| = Estimated Cost Share Amount: |  |  |  |

Signatures

| Participant | Date |
| --- | --- |
| NRCS | Date |
| Farm Services Agency | Date |
| SWCD | Date |

##### Job Sketch or Map

An aerial view or photo of the area can be shown here.

The U.S. Department of Agriculture (USDA) prohibits discrimination in all its programs and activities on the basis of race, color, national origin, sex, religion, age, disability, political beliefs, sexual orientation, or marital or family status.  (Not all prohibited bases apply to all programs).  Persons with disabilities who require alternative means for communication of program information (Braille, large print, audiotape, etc.) should contact USDA's TARGET Center at 202-720-2600 (voice and TDD).

To file a complaint of discrimination, write USDA, Director, Office of Civil Rights, Room 326-W, Whitten Building, 14th and Independence Avenue, SW, Washington, DC 20250-9410 or call (202) 720-5964 (voice and TDD).  USDA is an equal opportunity provider and employer.